7-4 )  PROCEDURE FOR BEAD WIRE MANUFACTURING

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANING | - Examine the Production plan - Report current material stock to planning dept | APM//LEA/SUP ,, | PC-PLAN-02D |
| MATERIAL RECEIIVING | - Receive quality-approved materials from mixing plant and stores. - Approved Rubber Compound - Approved Steel Wire - Plaster roll | APM//LEA/SUP | IT-FORM-02A IT-REC-054 IT-FORM-01 |
| MACHINE PREPARATION | - Check the Machine - Set the Machine as per Instruction | LEADER/OPE ,, | MC-CL-001 WI-001 |
| STEEL WIRE ROLL FIXING & RUBBERRIZING | - Fix steel wire roll to stand - Feed compound to extruder machine - Rubberize - Plastering with appropriate color - Dip in the soap bath (Grade “A” Wire) - Allow to dry - Fill the record sheet | OPE/HELPER HELPER OPE/HELPER ,, ,, ,, HELPER | WI-001 ,, ,, ,, ,, ,, PC-001 |

| Flow | Activity |  |  | Responsibility | Ref |  |
| --- | --- | --- | --- | --- | --- | --- |
| INSPECTION & GRADING STORING | - Check adequacy of rubberizing and proper plastering. - Check and record the circumference and weight of fist wire when starting & former changing. - Check the bead wire circumference and weight for each plant on random basis. - Send grade “A” bead wire to relevant location for storage - Do a quality check and approve the bead-wire production for each plant once a shift. |  |  | OPERATOR ,, LEADER ,, COP | PC-001 PC-001 COP-QC-004 |  |
| HAND OVER | - Send the approved bead wire lot to building section according to the bead wire requirement of the respective plant and update IFS system. |  |  | APM/LEADER | IFS System |  |
| REJECTION |  | - Report material wastage through IMI instruction | LEADER ,, |  | Scrapping Report |