7.5.1  Production for Air Bag Extrusion, Cellular Treads Extrusion.

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| RECEIVING MATERIAL | - Examine production plan - Check rubber compound received for adequacy and Quality | SUP/LED ,, | PC-PLAN-02E IT-FORM-02A WI-021 |
| EXTRUSION Rubberizing & Inspection | - Carryout extrusion according to the work instruction - Air bag extrusion - Cellular tread extrusion | OPE | WI-021 |
| INSPECTION | - Check the weight, diameter, thickness & perimeter (Air bag Extrusion - Check the weight, width, (Tread) thickness & perimeter (Cellular tread) | OPE OPE | AB-TABLE-01 WI-021 CT-TABLE-01 |
| DELIVERY | - Delivery to the relevant section | SUP/LED | PP-NOTE-02 |
| REJECTION | - Raise NC/IMI – Form - Report material wastage | SUP/QAM | NC-FORM-01 IMI-FORM-01 Scrapping Report |

7.5.2  Production for Air Bag Manufacturing.

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANING & CHECKINGMATERIAL | - Examine production plan - Examine available stocks and received materials. - Examine length, weight, thickness, - Outer perimeter relevant to type of air bag | SUP/LED ,, ,, OPE | PC-PLAN-02E E-Mail WI-021 |
| MAKING AIR BAG Rubberizing & Inspection | - Fix valve and apply rubber compound. - Apply solvent fix joint. - Press joint using pressing machine - Keep the bag for shaping | OPE ,, ,, ,, | WI-021 |
| PRESSING | - Check Mould temperature, steam pressure, air pressure hydraulic pressure. - Place green bag in press - Inflate bag when steam pressure is OK - Deflate and remove air bag after curing and record | OPE / LED OPE ,, ,, | AB-TABLE-02 WI-021 PC-014 |
| INSPECTION & HAND OVER | - Number the bag do post curing and send for the grading - Check air bag for any defects or air leak by re inflating | OPE / LED O / L /SUP | PP-NOTE-02 |
| GRADING | - Final product inspection & recording - Hand over to stores | OPE ,, | WI-033 PP-NOTE-02 |
| REJECTION | - Raise NC/IMI – Form - Report material wastage | SUP/QAM | NC--FORM-01 IMI-FORM-01 Scrapping Report |