7-6 )  PROCEDURE FOR CORD CALENDERING

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANING | - Examine the Production plan - Order Material from relevant department - Report current material stock to planning dept. | MAN/LEADER/OP LEADER/SUP LEADER/SUP | PC-PLAN-02F IFS-System Com.Soft Copy |
| MATERIAL RECEIIVING | - Receive nylon tyre cord from stores with test report - Receive Compounds from Mixing - Check lab report of tyre cords and topping compound. | LEADER/SUP LEADER/SUP LEADER/SUP | IT-REC-055 PC-SCHED-01 IT-REC-055 IT-FORM-02A WI-002 |
| MACHINE PREPARATION | - Check the Machine - Set Machine as per Instruction | LEADER/OPE ,, | MC-CL-002M MC-CL-002C WI-002 |
| RUBBERIZING | - Worm up and Issue the rubber compound to calender - Feed compound to calender machine - Rubberize - Check quality when rubberizing and record (Measure the thickness of canvas) - Complete Rubberizing, and remove rubberized cord roll form calender and attach identification tag - If necessary, cover cord roll with Polythene film. - Fill the record sheet and report rubberized qty to IFS System. | MILL OPE OPE/HELPER CORD OPE OPE/HELPER CORD OPE OPE-HELPER OPE/DATA ENTRY OPE | WI-002 WI-002 ,, PC-CAL-REC-01 ,, PC-002 IFS System |

7-6 )  PROCEDURE FOR BIAS CUTTING OF RUBBERIZED CANVAS

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| FEEDING MATERIALS | - Examine the Production plan - Feed rubberized canvas | LEADER/SUP/MAN OPE/HELPER | PC-PLAN-02F WI-003 |
| MACHINE PREPARATION | - Check the Machine - Set Machine Width & Angle as per specification | OPERATOR LEDER/OPE | MC-CL-004 PC-CAL-TABLE-03 |
| CUTTING & WINDING | - Check the current stock level available in the section and building section - Cut and remove any damages of the canvas - Cut joint and wind the canvas - Check width and weight, wrinkles when winding. - Attach identification sticker after completing the winding. - Hang onto the conveyer - Fill the record and report canvas production to IFS system | OPERATOR ,, ,, OPE/HELPER ,, ,, OPE/ DATA ENTRY OPE | WI-003 ,, ,, ,, ,, PC-004/ IFS System |
| DELIVARY | - Deliver canvas roll to the building section according to production plan after maturation period and enter details in to IFS system | LEADER/ HELPER | PC-PLAN-02F WI-003 |
| REJECTION | - Fill up IMI form and use NC form, if necessary - Weight and dispose damage canvas cuttings through the scraping report. - Analyze wastage graphically for improvement. | APM/LEADER/ APM/LEADER ,, | IMI-FORM NC-FORM-01 SRI-040 |

7-6 )  PROCEDURE FOR PROFILE CALENDERING

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANING & CHECKIG MATERIALS | - Examine the Production plan. - Check current stocks and inform to planning department - Check material coming from mixing section - Adequacy - Quality | OP/LEADER/SUP LEADER/SUP LEADER/SUP | PC-PLAN-02G Com.Soft Copy PC-SCHED-01 WI-005 IT-FORM-02A |
| CHECKING PREPARATION and if necessary use NC form | - Check the Machine. - Set Machine as per instruction | PROFILE OP/ MILL OP APM/LEADER/ OPE | MC-CL-005 MC-CL-002M WI-005 |
| PROFILE | - Worm up and issue compound - Profile calendaring | MILL OPERATOR PROFILE OPE | WI-005 ,, |
| INSPECTION & WINDING | - Check Wight, Thickness and width of tread and enter the record sheet - Attach identification sticker after winding complete. - Hang on the conveyer - Fill Record sheet and enter the tread & liner production qty to IFS system. | PROFILE OPE ,, HELPER OPE/DATA ENTRY OP | PC-005 ‘’ ,, PC-005 IFS System |
| DELIVARY | - Delivery to building section after the maturation period according to requirement. | HELPER | WI-005 |
| REJECTION | - Raise the IMI form - Report dispose compound to the IFS System and complete the scrapping report as per IMI instructions | LEADER/SUP ,, | IMI-FORM IFS System SRI-040 |