- INSPECTION OF INCOMING MATERIALS

a)  Materials from mixing plants & air bag section of SRI.

- Rubber Compounds

- Air Bags

b)  Materials from approved sources.

- Nylon Tyre Cords

- Steel wires

- Any other production related materials

(Kevlar bead wires, Stickers, Chafers)

c)  Procedure for inspection and verification.

All the incoming materials are subjected to a general inspection by the stores authorities at the point of unloading. At the time of receipt, COP officers are responsible to verify the compliance to the specification.

General inspection

- Whether the receiving quantity is equal to the order / invoiced quantity.

- Whether the packing comply with the packing/supply instruction.

- Whether there are any damages or irregularities.

- Quality certificate from the supplier/manufacture.

The receiving of materials to the stores is informed to the COP department over the phone with following details.

- Name of the materials and quantity.

- Manufacture or supplier.

For material from S.R.I, the test report from S.R.I laboratory is accepted. For material from any other sources, test samples are collected by COP department representative personally and send to S.R.I laboratory with required details. All the material should be kept under yellow tags until COP department acceptance is given.

8.2.1.1   Procedure for incoming inspection of materials, semi products prior to use.

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| RECEIVING MATERIAL | - General Inspection - Inform to the COP department about receipt of material. - Submit supplier quality certificate and laboratory approval to COP department. | Sup. Officer Sup. Officer Sup. Officer | COP- IM-WI-01 “ “ |
| ACCEPTANCE | - If lab reports are ok and if there are no any visual damages, authorize for use by placing “accepted “on test reports issued by SRI laboratory. - Reject if there are any visual damages and inform to COP manager. - Send the rejected material to the noncompliance area with red tag . - Return to the internal source of material with the IMI action and inform to the relevant authority. | COP OFFICER COP OFFICER COP OFFICER MGR (COP) | IT- FORM – 02/03/04 COP-IM-WI-01 COP-IM-WI-01 COP-IM-WI-01 IMI SYSTEM |

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| RECEING MATERIAL | - General inspection - Inform to the quality department about receipt of material. | Sup Officer Sup Officer | COP-IM-WI-01 “ |
| Sampling | - Receive the manufacture quality report from stores. - Collect samples from consignment after general inspection. - Fill up the sample data in the system. - Send the samples to the SRI laboratory. | COP OFFICER COP OFFICER COP OFFICER COP OFFICER | COP-IM-WI-01 COP-IM-WI(S)-01 IT-FORM-01 “ |
| TESTING | - Carry out the appropriate test at SRI laboratory. | MGR(LAB) | IT-REC-01C/01A |
| ACCEPTANCE | 1. Receive test reports from SRI laboratory - If lab reports are ok and if there are no any visual damages, authorize for use by placing “accepted” on test reports issued by SRI laboratory. - Reject if there are any visual damages or test reports are negative and inform to COP manager. - Send the rejected material to the noncompliance area with red tag. - Return to the internal source of material with the IMI action and inform to the relevant authority. | COP Officer COP Officer COP Officer COP Officer MGR (COP) | IT-REC-01C/01A COP-IM-WI-01 COP-IM-WI-01 COP-IM-WI-01 IMI SYSTEM |

8.2.1.2 Procedure for incoming inspection of material purchased from approved sources- Nylon Tyre cord & Steel wires.