8.3 PROCEDURE FOR GRADING OF BICYCLE TYRES, MOTOR CYCLE TYRES, LAWN AND GARDEN TYRES, AIR BAG, RIM TAPE AND TUBE

8.3.1 Procedure for grading of bicycle tyres. (Export & Domestic)

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANNING | - Examine the production plan and details of export orders. | QAM/QAO GRADER | PC-PLAN-02 CR-REP-05 |
| RECEIVING | - All types of tyres are received Over the conveyor - Unload in bunch of each presses - The count is recorded through the software - If necessary stacking relevant location | QAO/GRADER GRADER/ HELPER GRADE ,, | WI-011 ,, Com. System WI-011 ,, |
| INSPECTION RECORDING & REPORTING | - All the tyres are examined for any visual or technical defects and customer requirements. - Grade them in to two groups - Without defects - Pending for final grading - Re-grade B tyres in to A,C, D and record. - Prepare the daily rejection reports and inform to relevant section through the production soft system | GRADER ,, QAM /QAO LEADER LEADER/QAO/QAM | WI-011 PC-PLAN-02 CR-REP-05 Production Software ,, |
| FINISHING | - Finishing of different types of tyres are done according to the customer requirement. - Finished tyres are bundled according to the customer requirement | QAM/QAO/GRADER GRADER | CR-REP-05 WI-011 PC-PLAN-02 TSF-14-1 PC-PLAN-02 CR-REP-05 |
| HAND OVVER THE GOODS | - Grade A and grade C tyres hand over to the stores through System. - Grade “D” tyres scrap and hand over To relevant section | QAO/ASK GRADER GRADER / LEADER / QAO | FGH-REP-01 IFS System SCRAPPING REPORT(IFS) |

8.3.2  Procedure for grading of Lawn and garden tyres.

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANNING | - Examine the production plan and details of export orders. | QAM/QAO | PL-PLAN-02 CR-REP-05 |
| RECEIVING | - All types of tyres are received according to the transport schedule. - Accepted tyres stack in relevant location - The count is recorded in relevant record sheet | QAM/QAO GRADER GRADER HELPER GRADER | WI-013 TRP-SCHED-04 WI-013 WI-013/QC-019 |
| INSPECTION RECORDING AND REPORTING | - All the tyres are examined for any visual or technical defects and customer requirements. - Grade them in to two groups - Without defect - Pending for final grading - Re-grade B tyres in to A,C, D and record. - Prepare the daily rejection reports and inform to relevant section through the production soft system | QAM/QAO GRADERS GRADERS LEADER QAM/QAO LEADER/ GRADER/ QAO/ QAM | WI-013/CR-REP-05 /PC-PLAN-02 WI-013 Production Software ,, |
| AIR LEEK TEST | - Collect sample & carryout test - Tyres are stacked as lots, until confirmation of air leak test | QAM/QAO /GRADER QAO | WI-013/QC-021 ,, |
| FINISHING | - Finishing of different types of tyres are done according to the customer requirement. | QAM/QAO GRADER | CR-REP-05/WI-013 TSF-14-1 PC-PLAN-02A |
| # HANDING OVER THE GOODS | - Grade A and C tyres hand over to the stores through a System - Grade “ D” tyres , scrap and hand over to relevant section | QAO/ASK GRADE GRADER/ LEADE/QAO | FGH-REP-01 IFS System SCRAPPING REPORT(IFS) |

8.3.3  Procedure for Air bag grading.

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANNING | - Examine the production plan | QAM/QAO | PC-PLAN-02E |
| RECEIVING | - Examine the daily air bag handing over Report. - Receipt all types of air bags | GRADER, GRADER | Com. System |
| INSPECTION RECORDING AND REPORTING | - All air bags are examined for visual defects. - Grade them in to three groups - Air bag without defects - Air bag with defects - Defective Air bags (Unusable) - Hand over grade “B” level Air bag to repair. - Re-grade the acceptable Air bags in to “A” grade after repair. - Record test results and report to air bag section. | GRADER GRADER GRADERS ,, ,, | WI-033 ,, QC-026 |
| # FINISHING | - Attach information Label to the Air bag - Bundle, “A” grade Air bags. | GRADER GRADER | WI-033 |
| # HANDING OVER | - Hand over “A” grade Air bags to stores - Hand over “ D” grade Air bags for disposing. | QAO/GRADER QAO/GRADER | IFS System SCRAPPING REPORT |

8.3.4 PROCEDURE FOR GRADING OF MOTOR CYCLE TYRES.

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANNING | - Examine the production plan and details of export orders. | QAM/QAO GRADDER | PC-PLAN-02 CR-REP-05 |
| RECEIVING | - All types of tyres are received according to the transport schedule. - The count is record in relevant record book. | QAM /QAO GRADER LEADER | WI-012 TRP-SCHED-04 IT-MC-REC-01 |
| INSPECTION &RECORDING AND REPORTING | - All the tyres are examined for any visual or technical defects and customer requirements. - Grade them in to two groups - Without defects - Pending for final grading - Re-grade B tyres in to A, C and D and record. - Prepare the daily rejection reports and inform to relevant section through the production soft system | GRADER GRADER QAM/QAO LEADER/GRADER ,, | WI-013/CR-REP-05/ PC-PLAN-02 WI-013 WI-013 Production Software ,, |
| # FINISHING | - Pack tyre according to the customer requirement (Wrap or without wrap) | WRAPPER ,, | WI-013 CR-REP-05 |
| # HANDING OVER | - Hand over all grade A and C tyres to stores through IFS System - All “D” tyres scrap and handover to Relevant section | QAO/ASK LEADER/ QAO/GRADER | FGH-REP-01 SCRAPPING REPORT (IFS) |

8.3.5. Procedure for grading, finishing and final inspection of tubes

8.3.5.1.A Procedure for grading and finishing of tube (Moulded).

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANNING | - Examine production plan and details of export order - Take action to obtain material for the grading section | QAM/QAO/QAEX./ OPERATOR OPERATOR/QAO | PC-PLAN-02 CR-REP-05 PC-PLAN-02 CR-REP-05 |
| RECEIVING & STACKING | - All type of tube crates are received according to transport schedule. - Keep the records for unloaded crates - Stack the crates in relevant stacking area. | OPERATOR/ LEADER OPERATOR/ LEADER OPERATOR | TR-SHED-01 TU-IT-001 TU-WI-011-1 |
| VALVE PART FIXING & INFLATING | - Recheck the tube quantity of the crates. - Keep the records - Check tube quantities with receiving report. - Fix the valve part which need to inflate tube - Inflate tube according to relevant gauge and stack them in a trolley for final inspection and keep the records. | OPERATOR ,, QAO/LEADER OPERATOR OPERATOR | TU-WI-011-1 TU-IT-001 TU-IT-001 PP-NOTE-02 TU-WI-011-1 TU-IT-001 TU-WI-011-1 |

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| INSPECTION & RECORDING | - After relevant maturation period all tubes are examined for any visual or technical defects. (Maturation period, minimum 8hr) - Grade them in two groups. - Without Defect - Pending for Final Inspection - Re grade B tube either A, or D and record. - Defects are entered to the software | OPERATOR OPERATOR QAO/LEADER ,, | TU-WI-011-2 TU-WI-011-2 TU-WI-0011-4 TUBE PRODUCTION SOFTWARE |
| DEFLAITING | - Deflate the all A tube which are completed the relevant time period. (minimum 8 hr) - Deflated tube during that time are sent for re inspection. | OPERATOR OPERATOR | TU-WI-011-3 TU-WI-011-3 |
| PRE PACKING | - Do necessary thing (printing, filling sludge, sealant or latex…etc) according to customer requirements - Pack tubes with polypropylene bag or rack for relevant market or customer with a relevant tag - Maintain the record. | OPERATOR OPERATOR OPERATOR | TU-WI-011-5/9/10 TU-WI-011-5 IT-TU-REC-01 |
| HAND OVER | - Grade A tube stack for packing - Grade D tubes scrap and handover relevant section. | OPERATOR OPERATOR | TU-WI-011-5 SCRAPING REPORT (IFS) |

8.3.5.1.B Procedure for grading and finishing of tube and Rim tape (Autoclave).

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANING | - Examine production plan and details of export order - Take actions to obtain material for the grading section. | QAM/QAO OPERATOR OPERATOR/QAO | PC-PLAN-02 CR-REP-05 PC-PLAN-02 CR-REP-05 |
| RECEIVING & STACKING | - All type of tube and Rim tape crates are received according to transport schedule - Keep the records for unload crates - Stack the crates in relevant inspection line. | OPERATOR/ LEADER OPERATOR/ LEADER OPERATOR | TR-SHED-01 TU-IT-001 TU-WI-011-7/8 |
| INSPECTION of VALVE PARTS FIXING, INFLATING | - Recheck the tube and rim tape quantity of the crates. - Keep the records - Check the quantities with handover report - All tubes and Rim tape are examined for any visual or technical defects - Grade them in two group. - Without defects - Pending for final inspection - Fix relevant valve parts to A tubes and Rim tape where necessary - Inflate air (Tubes only) and record. - Re grade B tube and Rim tape in to A, D and record - Defects enter to the software | OPERATOR ,, ,, ,, ,, ,, ,, LEADER/QAO LEADER/QAO | TU-WI-011-7/8 TU-IT-001 PP-NOTE-02 TU-IT-001 TU-WI-011-7/8 TU-WI-011-7/8 TU-WI-011-7/8 TU-WI-011-7/8 TUBE PRODUCTION SOFTWARE |

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| DEFLATING | - Deflate the all tube which are completed the relevant time period (minimum 08 hr ) - Air reduction tubes are sent for re inspection. | OPERATOR OPERATOR | TU-WI-011-7 TU-WI-011-7 |
| PRE-PACKING | - Print all tubes and rim tape with relevant mark - Do necessary thing (applying silicon only for rim tape) rolling & packing (only for rim tape) according to customer requirements. - Pack tubes with rack or bag for relevant market or customer with a relevant tag - Pack Rim tape according to customer requirements - Maintain records. | OPERATOR OPERATOR ,, ,, ,, | TU-WI-011-7/8 TU-WT-011-8 TU-WT-011-7/8 TU-WT-011-7/8 IT-TU-REC-01 |
| HAND OVER | - Grade A Tubes Stack for packing - Grade D tubes scrap and handover relevant section | LEADER/QAO QAO/OPERATOR | TU-WI-011-7/8 SCRAPIN REPORT (IFS) |

8.3.5.1.C Procedure for grading and finishing of tube (Jointed)

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANNING | - Examine production plan and details of export order - Take actions to obtain material for the grading section | QAM /QAEx/QAO OPERATOR OPERATOR / QA | PC-PLAN-02 CR-REP-05 PC-PLAN-02 |
| RECEIVING & STACKING | - All type of tube crates is received according to transport schedule - Keep the records for unload crates. - Stack the crates in relevant stacking area. | OPERATOR ,, ,, | TR-SHED-01 TU-IT-001 TU-WI-011-6 |
| CHECKING & VALVE PART FIXING | - Recheck the tube quantity of crates and records - Examined all tubes for any visual or technical defects - Grade them in two group. - Without defect - Pending for final inspection - Fix relevant valve parts to A tubes and Inflate air and stack in trolleys and record. - Re grade B tube in to A and D. - Defects enter to the software | OPERATOR OPERATOR ,, ,, LEADER/QAO LEADER/QAO | TU-IT-001 TU-WI-011-6 TU-WI-011-6 TU-IT-001 TUBE PRODUCTION SOFTWARE |

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| DEFLATING | - Check the valve - Deflate the tubes which have no air reduction - Deflated tubes are sent for re inspection | OPERATOR ,, ,, | TU-WI-011-6 ,, ,, |
| PRE-PACKING | - Print all tubes with relevant mark - Pack tubes with polypropylene bag or stack on racks for relevant market or customer with the appropriate tag | OPERATOR ,, | TU-WI-011-6 ,, |
| HAND OVER | - Stack grade A tube for packing - Grade D tubes scrap and handover to relevant section. | QAO/OPERATOR ,, | TU-WI-011-6 SCRAPING REPORT (IFS) |

8.3.5.3 Procedure for final inspection of tubes

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| INSPECTION & NON DESTRUCTIVE TEST | - Collect samples for testing according to sampling plan - Carryout non-destructive test - Record test results | OPERATOR ,, ,, | TU-TM-002 TU-TM-002 TU-TM-004 TU-IT-011 |
| DESTRUCTIVE TEST | - Collect samples for destructive test - Carryout destructive test - Record test results | OPERATOR ,, ,, | TU-TM-002 TU-TM-002 TU-TM-004 TU-IT-010 TU-IT-011 |

8.3.5.4 PROCEDURE FOR PACKING OF TUBES.

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| PLANING | - Examine the production plan and details of export orders. - Take actions to obtain material Receiving | QAM/QAO/QAEx:/ OPERATOR OPERATOR QAO/QAEx: | PC-PLAN-02 CR-REP-05 PC-PLAN-02 CR-REP-05 |
| RECEINING & RECORDING | - All types of tubes are received from the tube grading | QAM/QAO/QAEx:/ OPERATOR | TU-WI-011-11 |
| PACKING & INSPECTION | - Packing, Inspection and finishing | OPERATOR | TU-WI-011-11 TU-IT-005 TU-IT-003 |
| HANDING OVER | - Hand over all grade A Tubes to stores | QAM/QAO/QAEx:/ OPERATOR | FGH-REP-01 |