- PROCEDURE FOR PRE SHIPMENT INSPECTION

- SCOPE

This procedure is applicable to the product Quality inspection for Quality assurance of the shipment under established customer AQL value, prior to the delivery.

- RESPONSIBILITY

Head of Pre-Shipment Inspection.

- PROCEDURE FOR PRE SHIMENT INSPECTION

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| Weekly Order Details | - Follow the Order Plan Reports. - Follow The ASM & OEM Loading Schedule. - Check with the IFS System for the pending items of the orders. - Check the customer requirements with the detail sheets - . Check with PSI Internal References - Check & Compare monthly Order plan reports | IM Acc. Mgs ,, Head of the PSI IM Deps PSI Member PSI Officers | IFS SYSTEM E mail Detail sheet through Email (if have pending order) PSI/REC/001 PSI/REC/002 IFS SYSTEM E mail |
| Documentation Clarification | - Request the Orders Related to PO Sheet. - Request any customer required Documents. - Make customer Packing List available | Customer /IM Dept. IM Dept. Shipping Dept. | Email Email IFS /Email |
| Sample Selection | - Select the Samples as per the established Sampling plan. - Decide the sample status as per the acceptance and rejection criteria under given AQL level. | PSI Officers | Sampling Plan IT/001 |

| Flow | Activity | Responsibility | Ref |
| --- | --- | --- | --- |
| Pre Shipment Inspection | - Check Reference Master sample if applicable. - Check & conform, documented information and sample details are correct. - Check the Visual defects and Identify the defect category. - If it is specified by the customer, use the derogation form as and when required - Check to customer special marking or required Stickers. - Identify the defect category Critical, Major, Minor | R&D / IM / Customer ,, PSI Officer. ,, ,, | Master Sample AQL Table Details Sheet /PO WI-040 |
| Report Preparation & Decision on | - Completion of the Inspection reports. - Inform the inspection results to the relevant department for the Shipment conformation, Rejection or Hold and Re-grading. - Inform the Findings and Inspection status to the head of pre shipment inspection. - Update the IFS system. - Establish the Pre Shipment Inspection status. - Accept (pass) the shipment If the lot is conforming to customer AQL level, or Reject or Hold and Re-grade not conforming the customer AQL. - Obtain the top management approval in case it is necessary. | PSI Officers PSI Member PSI Officers PSI Officers/ IM Dept./ Customers. | PSI/REC/001 ,, IFS System PSI/REC/001 & Customer documents |